| Key Management Test Cases |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Case ID | Category | Test Case Description | Test Steps | Pass/Fail |
| TCKM001 | 4.6.3.8.1 Get Current Primary Key | If the Find My network pairing is complete, verify the accessory responds with Get_Current_Primary_Key_Response with Current Primary Key. | Perform this test case with the Find My app and FMCA (Find My Certification Assistant) app using the steps below. 1. From iOS device A, go to Settings, select Apple ID on top of page, select Find My, and set “My Location” to this device. Please note: ensure that this Apple ID is only logged into iOS device A. 2. Launch the Find My app. Under “Items," select Add Item, select Other Supported Item, and select Connect. Name the item and select Continue to complete the setup to add to Find My. 3. Launch the FMCA app. Select the accessory under My Items. Please note: accessories that were unable to successfully pair to a device in Find My will be listed under the “My Items” section as “Find My Pairing issues." 4. Select “Key Management” under the Topics section. 5. Select “Verify Primary Key” under the Topics section to run the test. 6. Verify results are passing. 7. Under the Results section, select “Save Results." |  |
| TCKM002 | 4.6.3.4.9 Keyroll Indication | Verify that the Keyroll_indication opcode is used by the accessory to indicate a primary key roll has occurred. | Perform this test case with the Find My app and FMCA (Find My Certification Assistant) app using the steps below. A Debug control point enabled sample is required to perform this test case. 1. From iOS device A, go to Settings, select Apple ID on top of page, select Find My, and set “My Location” to this device. Please note: ensure that this Apple ID is only logged into iOS device A. 2. Launch the Find My app. Under “Items," select Add Item, select Other Supported Item, and select Connect. Name the item and select Continue to complete the setup to add to Find My. On the iOS device, go to Settings, select Display & Brightness, and set Auto-Lock to “Never.” 3. Launch the FMCA app. Select the accessory under My Items. Please note: accessories that were unable to successfully pair to a device in Find My will be listed under the “My Items” section as “Find My Pairing issues." 4. Select “Key Management” under the Topics section. 5. Select “Set+Verify Key Rotations” under the Topics section to run the test. 6. Verify results are passing. 7. Under the Results section, select “Save Results." |  |
| TCKM003 | 6.3.3.1 After pairing | Verify that the accessory uses the Primary key Pi (where i=1) as a BLE advertisement and enters NearOwner mode | Perform this test case with the Find My app and FMCA (Find My Certification Assistant) app using the steps below. 1. Launch the Find My app. Under “Items," select Add Item, select Other Supported Item, and select Connect. Name the item and select Continue to complete the setup to add to Find My. 2. Launch the FMCA app. 3. Under “Discovered Items," verify accessory name is listed and is in a Nearby state. Alternatively, from a second iOS device B (on a different Apple ID account), launch the FMCA app. Under “Discovered Items," verify the accessory is listed as “Nearby." |  |
| TCKM004 | 6.3.2 Key sequences and rotation policy | Verify the accessory generates public key sequences with different key rotation intervals, referred to as primary and secondary keys. Please note: steps 5-6 and 11-12 can done with any tests as long as there is a 24 hour time gap between test execution. | Perform this test case with the Find My app and FMCA (Find My Certification Assistant) app using the steps below. 1. From iOS device A, go to Settings, select Apple ID on top of page, select Find My, and set “My Location” to this device. Please note: ensure that this Apple ID is only logged into iOS device A. 2. Launch the Find My app. Under “Items," select Add Item, select Other Supported Item, and select Connect. Name the item and select Continue to complete the setup to add to Find My. 3. Launch the FMCA app. Select the accessory under My Items. Please note: accessories that were unable to successfully pair to a device in Find My will be listed under the “My Items” section as “Find My Pairing issues." 4. Select “Key Management” under the Topics section. 5. Select “Verify Primary Key” under the Topics section to run the test. 6. Verify results are passing. 7. Under the Results section, select “Clear Results." 8. Wait 24 hours. 9. Launch the FMCA app. Select the accessory under My Items. 10. Select “Accessory Information” under the Topics section. 11. Select “Get Accessory Information” under the Topics section to run the test. 12. Verify results are passing. 13. Under the Results section, select “Save Results." 14. Select “< Accessory Name” to go to the previous screen. 15. Select “Share Results." 16. Follow instructions to share results to your computer. 17. Results are saved as a JSON file. Use any JSON tool to reformat the results. 18. Locate “additionalInfo” field. 19. Verify the follow Primary key index resolved is >1 and secondary key index resolved is >1. |  |