| NFC Test Cases |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Case ID | Category | Test Case Description | Test Steps | Pass/Fail |
| TCN001 | 3.3.4 Serial Number Lookup / 8.3 Implementation | Verify the accessory is readable through NFC and advertises the correct payload when paired. | Perform this test case with the Find My app and FMCA (Find My Certification Assistant) app using the steps below. 1. Launch the Find My app. Under “Items," select Add Item, select Other Supported Item, and select Connect. Name the item and select Continue to complete the setup to add to Find My. 2. Launch the FMCA app. Select the accessory under My Items. Please note: accessories that were unable to successfully pair to a device in Find My will be listed under the “My Items” section as “Find My Pairing issues." 3. Select “NFC” under the Topics section. 4. Select “Paired Information” under the Topics section to run the test. 5. Hold the iOS device over the center of the NFC tag on the accessory. 6. Verify the paired information returned. 7. Verify results are passing and the following information is returned - battery health, Product ID, S/N look up method, FW version, and URL. For Encrypted serial number, copy and paste the weblink to a text document and verify the URL contains character string after “e=." 8. Verify Product Data and FW version returned match the information provided in the Accessory Information service. 9. Under the Results section, select “Save Results." |  |
| TCN002 | 3.3.4 Serial Number Lookup / 8.3 Implementation | Verify the accessory is readable through NFC and advertises the correct payload when unpaired. | 1. While the accessory is unpaired and out of its packaging, hold the iOS device over the center of the NFC tag on the accessory. 2. Once prompted, open the URL link. 3. Verify the unpaired accessory payload URL matches the following format: https://found.apple.com/accessory?pid=%04x&b=%02x&fv=%08x&bt=%s&sr=%s 4. Copy and paste the weblink to a text document and verify the URL contains character string after “pid=," “b=," “fv=," “bt” and “sr=.” |  |