| Firmware Updates Test Cases |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Case ID | Category | Test Case Description | Test Steps iOS versions older than iOS 17 | Pass/Fail |
| TCFW001 | 4.5.3.6. Firmware Version | Verify that the characteristic value changes after every firmware update. | Perform this test case with the Find My app using the steps below. Applies to accessories that support Firmware Update by UARP or by accessory app. For Firmware Update by UARP, please download and install the Find My Network Configuration Profile from the MFi Portal Content Center. 1. Launch the Find My app. Under “Items," select Add Item, select Other Supported Item, and select Connect. Name the item and select Continue to complete the setup to add to Find My. 2. Launch the FMCA app. Select the accessory under My Items. Please note: accessories that were unable to successfully pair to a device in Find My will be listed under the “My Items” section as “Find My Pairing issues." 3. Select “Firmware” under the Topics section. 4. Select “Firmware Version” under the Topics section to run the test. 5. Verify firmware revision is in the following format: x[.y[.z]] and select Clear Results. 6. If the accessory supports Firmware Update by UARP, please follow steps 7-12 to complete firmware update. If accessory supports firmware update by the accessory app, perform the firmware update and skip to step 14. 7. Launch the FMCA app. Select the accessory under My Items. 8. Select “Firmware Settings." 9. Select “Choose a File…." 10. Select a .UARP file that is stored locally on the iOS device. i.e Files folder 11. Launch the Find My app, select the accessory under My Items, and scroll down to the accessory information. 12. Select “Update available." 13. Select “Update” to the pop up prompt. 14. Open the FMCA app. 15. Select “Firmware” under the Topics section. 16. Select “Firmware Version” under the Topics section to run the test. 17. Verify firmware revision is in the following format: x[.y[.z]] and has incremented from step 5. |  |
| TCFW002 | 10. Firmware Update | Verify the Find My accessory can handle an interruption during the firmware update process when a user cancels/aborts. | Perform this test case with the Find My app and FMCA (Find My Certification Assistant) app using the steps below. Applies to accessories that support Firmware Update by UARP. Please download and install the Find My Network Configuration Profile from the MFi Portal Content Center. 1. Launch the Find My app. Under “Items," select Add Item, select Other Supported Item, and select Connect. Name the item and select Continue to complete the setup to add to Find My. 2. Launch the FMCA app. Select the accessory under My Items. Please note: Accessories that were unable to successfully pair to a device in Find My will be listed under the “My Items” section as “Find My Pairing issues." 3. Select “Firmware” under the Topics section. 4. Select “Firmware Version” under the Topics section to run the test. 5. Note down firmware revision and select Clear Results. 6. Select the accessory name on top left to return to the previous screen. 7. Select “Firmware Settings." 8. Select “Choose a File…." 9. Select a .UARP file that is stored locally on the iOS device. i.e Files folder 10. Select “Update available." 11. Select “Cancel” to the pop up prompt. 12. Select the accessory name on top left to return to the previous screen. 13. Select “Firmware” under the Topics section. 14. Select “Firmware Version” under the Topics section to run the test. 15. Verify the Firmware version remained the same from step 5. |  |
| TCFW003 | 10. Firmware Update | Verify the Find My accessory can handle an interruption during the firmware update process when the accessory is Out of Range. | Perform this test case with the Find My app and FMCA (Find My Certification Assistant) app using the steps below. Applies to accessories that support Firmware Update by UARP. Please download and install the Find My Network Configuration Profile from the MFi Portal Content Center. 1. Launch the Find My app. Under “Items," select Add Item, select Other Supported Item, and select Connect. Name the item and select Continue to complete the setup to add to Find My. 2. Launch the FMCA app. Select the accessory under My Items. Please note: accessories that were unable to successfully pair to a device in Find My will be listed under the “My Items” section as “Find My Pairing issues." 3. Select “Firmware” under the Topics section. 4. Select “Firmware Version” under the Topics section to run the test. 5. Note down firmware revision and select Clear Results. 6. Select the accessory name on top left to return to the previous screen. 7. Select “Firmware Settings." 8. Select “Choose a File…." 9. Select a .UARP file that is stored locally on the iOS device. i.e Files folder 10. Select “Update available." 11. Select “Update” to the pop up prompt. 12. Bring the iOS device out of range of the accessory. 13. Bring the iOS device back into of range of the accessory. 14. Verify the accessory reconnects with the iOS device. 15. From the Find My app, select Play Sound. 16. Verify sound is heard from the accessory. 17. From the FMCA app, select “Firmware” under the Topics section. 18. Select “Firmware Version” under the Topics section to run the test. 19. Verify the Firmware version remained the same from step 5. |  |