| Beacon States Test Cases |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Case ID | Category | Test Case Description | Test Steps | Pass/Fail |
| TCB007 | 2.7.2 Connected | Verify that the accessory supports simultaneous connections to two Apple devices on the same Apple ID account. | Perform this test case with the Find My app using the steps below. 1. From iOS device A, launch the Find My app. Under “Items," select Add Item, select Other Supported Item, and select Connect. Name the item and select Continue to complete the setup to add to Find My. 2. From a second iOS device B, log into the same Apple ID account as device A. 3. From iOS device A, open the Find My app. Under “Items," select the accessory, and select Play Sound. *Applies to accessories that support sound maker. 4. Verify sound is heard from the accessory. *Applies to accessories that support sound maker. 5. Wait for sound to complete. 6. From iOS device B, open the Find My app. Under “Items," select the accessory, and select Play Sound. *Applies to accessories that support sound maker. 7. Verify sound is heard from the accessory. *Applies to accessories that support sound maker. 8. Sign out of Apple ID account iOS device B 9. Disable Bluetooth on iOS device A from the iOS settings. 10. From iOS device B, log into the same Apple ID account as device A. 11. From iOS device B, open the Find My app. Under “Items," select the accessory, and select Play Sound. *Applies to accessories that support sound maker. 12. Verify sound is heard from the accessory. *Applies to accessories that support sound maker. 13. Enable Bluetooth on device A on the iOS settings. 14. From iOS device A, open the Find My app. Under “Items," select the accessory, and select Play Sound. *Applies to accessories that support sound maker. 15. Verify sound is heard from the accessory. *Applies to accessories that support sound maker. |  |
| TCB008 | 2.7.4 Separated | Verify that the accessory enters Separated state when it is paired and boots up from a power cycle or other re-initialization procedure | Perform this test case with the Find My app and FMCA (Find My Certification Assistant) app using the steps below. 1. From iOS device A, launch the Find My app. Under “Items," select Add Item, select Other Supported Item, and select Connect. Name the item and select Continue to complete the setup to add to Find My. 2. Launch the FMCA app. Select the accessory under My Items. Please note: accessories that were unable to successfully pair to a device in Find My will be listed under the “My Items” section as “Find My Pairing issues." 3. Disable Bluetooth on device A from the iOS settings. 4. Power cycle the accessory (if supported) or disable and enable the Find My network. 5. From a second iOS device B (on a different Apple ID account), launch the FMCA app. Under “Discovered Items," verify the accessory is listed as “Separated." 6. Enable Bluetooth on device A on the iOS settings. 7. From iOS device A, launch the FMCA app. Under “Discovered Items," verify the accessory is listed as “Nearby.” Alternatively, from a second iOS device B (on a different Apple ID account), launch the FMCA app. Under “Discovered Items," verify the accessory is listed as “Nearby." * Swipe down from the top of the screen to refresh the FMCA app. 8. From device A, launch the Find My app. 9. Under “Items," select the accessory, and select Play Sound. *Applies to accessories that support sound maker. 10. Verify sound (or vibration) is heard. *Applies to accessories that support sound maker. |  |