| General Requirements Test Cases |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Case ID | Category | Test Case Description | Test Steps | Pass/Fail |
| TCG001 | 3.2 General | Verify accessory is listed as a qualified end product by the Bluetooth SIG | 1. Verify provided BT Sig# information on certificate (or BT Sig website - https:// www.bluetooth.org/en-us/test-qualification/qualification-overview) matches accessory Product Plan (Questionnaire) information. |  |
| TCG002 | 3.3.5 Find My network disable | Verify that the accessory has a physical mechanism to disable Find My network (e.g, power off button, battery removal) based on user intent. | Perform this test case with the Find My app and FMCA (Find My Certification Assistant) app using the steps below. 1. From iOS device A, launch the Find My app. Under “Items," select Add Item, select Other Supported Item, and select Connect. Name the item and select Continue to complete the setup to add to Find My. 2. Physically disable the Find My network on the accessory (e.g, power off button, battery removal). 3. From a second iOS device B (on a different Apple ID account), launch the FMCA app. Under “Discovered Items," verify the accessory is not listed. 4. From iOS device A, launch the Find My app. Under “Items," select the accessory, and select Play Sound. 5. Verify response “Item Not Reachable” is returned. 6. Enable the Find My network. 7. Select Play Sound. 8. Verify sound (or vibration) is heard. |  |
| TCG003 | 3.3.4 Serial number lookup | Verify that the serial number is etched, engraved or directly printed on the accessory. Verify that the serial number is easy to find on the accessory and that it is unique to each sample. | 1. Verify that the serial number is etched, engraved or directly printed on the accessory and that it is unique to each sample. |  |
| TCG004 | 3.3.2 Product specific requirements | Verify that an accessory that is not “easily discoverable” includes a sound maker and a motion detector to support motion-triggered UT sound alerts. | 1. Verify accessory includes a sound maker and a motion detector if it meets all of the following criteria. - The accessory is ≤ 30 cm in at least one dimension. - The accessory is ≤ 18 cm X 13 cm in two of its dimensions. - The accessory is ≤ 250 cm3 in three dimensional space. |  |
| TCG005 | 3.3.3 Find My mark | The Apple-provided location icon or the Locate with Apple Find My badge is required on all Licensed Products unless any one of these three criteria are met: • The Licensed Product always requires Bluetooth pairing before its intended use, and has implemented Find My device naming that indicates it is currently trackable by its owner. For example, Bluetooth wireless headphones meet this criteria. • The Licensed Product is a dedicated tracking device which has no purpose other than to locate it using Apple Find My, and it has a motion detector and sound maker. For example, a locator fob that includes motion detection and sound, and is designed to attach to other objects would meet this criteria. • The Find My network-enabled Licensed Product is a personal computing device which has internet connectivity that is apparent to the end-user. For example, computers, tablets, and digital watches that offer internet connectivity meet this criteria. | 1. Verify the accessory includes an Apple-provided location icon or the Locate with Apple Find My badge that meets the requirements as indicated in the Works with Apple Find My Identity Guidelines. |  |
| TCG006 | 4.5 Accessory Information service / 4.5.3 Characteristics | Verify that the accessory supports the Accessory Information service (Primary) and includes mandatory characteristics. | Perform this test case with the Find My app and FMCA (Find My Certification Assistant) app using the steps below. 1. Launch the Find My app. Under “Items," select Add Item, select Other Supported Item, and select Connect. Name the item and select Continue to complete the setup to add to Find My. 2. Launch the FMCA app. Select the accessory under My Items. Please note: accessories that were unable to successfully pair to a device in Find My will be listed under the “My Items” section as “Find My Pairing issues." 3. Select “Accessory Information” under the Topics section. 4. Select “Get Accessory Information” under the Topics section to run the test. 5. Verify the results include the following characteristics - Product Data: Verify that the Product Data value matches the value assigned to this accessory's Product Plan on the MFi Portal. - Manufacture Name: Verify the manufacturer name matches the brand on the accessory. - Model Name: Verify the model name matches the manufacturer specific model of the accessory. - Accessory Category: Verify the category value set closely resemble the physical product. If none of the accessory categories provided match the physical product, “Other” (128) must be chosen. - Accessory Capabilities: Verify the capabilities claimed are supported on the accessory. - Verify Reserved characteristic is missing. 6. For accessories that support serial number lookup over BLE. Perform the following step. Select “Get Serial Number” under the Topics section to run the test. For accessories that support serial number lookup over NFC only, skip to step 9. 7. Place accessory into a serial number read state. 8. Verify test completed with a green check. 9. Under the Results section, select “Save Results." 10. Select “< Accessory Name” to go to the previous screen. 11. Select “Firmware” under the Topics section. 12. Select “Firmware Version” under the Topics section to run the test. 13. Verify firmware revision is in the following format: x[.y[.z]] and matches the value listed on the Find My Network Compliance Questionnaire. 14. Select “Find My Version” under the Topics section to run the test. 15. Verify Find My Version is set to 1.0.0. 16. Under the Results section, select “Save Results." 17. Select “< Accessory Name” to go to the previous screen. 18. Select “Power” under the Topics section. 19. Select “Battery Type” under the Topics section to run the test. 20. Verify the capabilities claimed are supported on the accessory. i.e., Powered, Non rechargeable battery, or Rechargeable battery 21. Select “Battery Level” under the Topics section to run the test. 22. Verify the current battery level is returned. 23. Under the Results section, select “Save Results." |  |
| TCG007 | (R2 addendum R1) 2. Accessories that advertise non-Find My network Bluetooth payload | When connected to hosts that are not paired to the Find My network, the accessories shall expose only paired owner information characteristic of the Find My network service. | Perform this test case with the FMCA app using the steps below. This test applies to accessories that advertise a non-Find My network Bluetooth Payload. 1. Pair to the accessory over Bluetooth. 2. Use the accessory for its primary purpose. 3. Launch the FMCA app. Under “Discovered Items," verify the accessory is not listed as “Nearby” or “Separated”. |  |