| General Requirements Test Cases |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Case ID | Category | Test Case Description | Test Steps | Pass/Fail |
| TCG008 | 3.2 General | Verify accessory does not operate simultaneously on the Find My network and other finder network, or implement functionality which may interfere with the security and privacy requirements referenced in the Find My Accessory Specification. | Perform this test case with the Find My app and manufacturer app (if applicable) using the steps below. 1. Launch the Find My app. Under “Items," select Add Item, select Other Supported Item, and select Connect. Name the item and select Continue to complete the setup to add to Find My. 2. Launch the manufacturer’s app. 3. Verify accessory does not operate simultaneously on the Find My network and other finder network, or implement functionality which may interfere with the security and privacy requirements referenced in the Find My Accessory Specification. 4. From the Find My app, select Play Sound. Verify sound is audible. 5. From the manufacturer’s, adjust accessory’s setting, if applicable. 6. From the Find My app, select Play Sound. Verify sound is audible. |  |
| TCG009 | 5.1.2 Payload for nearby state | An accessory must always advertise the Find My network payloads once every 2 seconds. | Perform this test case with the Find My app and a Bluetooth sniffer using the steps below. 1. From iOS device A, launch the Find My app. Under “Items," select Add Item, select Other Supported Item, and select Connect. Name the item and select Continue to complete the setup to add to Find My. 2. Disable Bluetooth from the iOS settings. Perform the next step within 15 minutes while accessory is in the Nearby state. 3. Using a Bluetooth sniffer, verify the Find My network Bluetooth Advertising Interval is 2 seconds. |  |
| TCG010 | 4.6.3.4.2 Persistent connection status | Verify the accessory advertises using an advertising interval of 30ms for a duration of 3 seconds if persistent connection is enabled, on a link lost event. | Perform this test case with the Find My app, FMCA app, and a Bluetooth sniffer using the steps below. A Debug control point enabled sample is required to perform this test case. 1. From iOS device A, go to Settings, select Apple ID on top of page, Select Find My, and set “My Location” to this device. Please note: ensure that this Apple ID is only logged into iOS device A. 2. From iOS device A, launch the Find My app. Under “Items," select Add Item, select Other Supported Item, and select Connect. Name the item and select Continue to complete the setup to add to Find My. 3. Launch the FMCA app. Select the accessory under My Items. 4. Select “Connectivity” under the Topics section. 5. Select “Verify Connection” under the Topics section to run the test. 6. Verify results are passing. 7. Select “Connection Status” under the Topics section to run the test. 8. Verify results are passing. 9. Disable Bluetooth from the iOS settings. 10. Using a Bluetooth sniffer, verify the Find My network Bluetooth Advertising Interval is 30 ms intervals for 3 seconds. |  |
| TCG011 | 3.3.1 Bluetooth | Verify the accessory supports the Tx Power service and that the Tx power level characteristic to the BLE EIRP at the antenna is ≥ +4dBm | Perform this test case with the Find My app and FMCA (Find My Certification Assistant) app using the steps below. 1. From iOS device A, launch the Find My app. Under “Items," select Add Item, select Other Supported Item, and select Connect. Name the item and select Continue to complete the setup to add to Find My. 2. Launch the FMCA app. Select the accessory under My Items. Please note: accessories that were unable to successfully pair to a device in Find My will be listed under the “My Items” section as “Find My Pairing issues." 3. Swipe down from the top of the screen to refresh the FMCA app. Under “Discovered Items," verify accessory is listed as “Nearby," and then select the accessory. Alternatively, from a second iOS device B (on a different Apple ID account), launch the FMCA app. Under “Discovered Items," verify the accessory is listed as “Nearby." 4. Under Tx Power Level, verify the TX Discovered Level is ≥ +4dBm and matches the MFi Expected value. |  |
| TCG012 | 5.1.4 Advertisement in low battery state | Low battery state should not cease Fine My advertisement (unless the device can no longer rotate its keys.) | Perform this test case with the Find My app and FMCA (Find My Certification Assistant) app using the steps below. 1. From iOS device A, launch the Find My app. Under “Items," select Add Item, select Other Supported Item, and select Connect. Name the item and select Continue to complete the setup to add to Find My. 2. Place the accessory in a low battery state. 3. Disable Bluetooth on device A from the iOS settings. 4. From a second iOS device B (on a different Apple ID account), launch the FMCA app. Under “Discovered Items," verify the accessory is not listed. 5. Enable Bluetooth on device A from the iOS settings. 6. From iOS device A, launch the FMCA app. Under “Discovered Items," verify the accessory not listed. |  |
| TCG013 | 3.3.4 Serial number lookup / 4.6.3.8.3 Get Serial Number | Verify that the serial number is only readable over BLE when paired and upon user action (e.g., press and hold a button). When a user puts the accessory into the serial number read state, verify the serial number is available look up for a 5 minute duration. If the accessory is not in serial number read state, it must send Command Response with response status 0xFFFF Invalid_command. Please note: this test case is only applicable to accessories that support serial number lookup over BLE. | Perform this test case with the Find My app and FMCA (Find My Certification Assistant) app using the steps below. 1. From iOS device A, launch the Find My app. Under “Items," select Add Item, select Other Supported Item, and select Connect. Name the item and select Continue to complete the setup to add to Find My. 2. Disable Bluetooth on device A from the iOS settings. 3. Wait 15 minutes before proceeding to the next step. Alternatively, power cycle the accessory (if applicable). 4. From a second iOS device B (on a different Apple ID account), launch the FMCA app. Under “Discovered Items," verify accessory is listed as “Separated," and then select the accessory. 5. Under “Non Owner Control Point," select “Present Unwanted Tracking Alert." 6. Select the “Get Serial Number” test, put the accessory into a serial number read state, and select “Send Command." On the second device start a stop watch timer. 7. Verify test completed with a green check. 8. Select “Done” on the top left corner of the window. 9. Wait for the timer to hit the 4 minute mark. Under “Discovered Items," verify accessory is listed as “Separated," and then select the accessory. 10. Under “Paired Owner Information," select “Get Serial Number” test. Do not put the accessory into a serial number read state. Select “Send Command." 11. Verify test completed with a green check. 12. Select “Done” on the top left corner of the window. 13. Wait for the timer to pass the 5 minute mark. Under “Discovered Items," verify accessory is listed as “Separated," and then select the accessory. 14. Under “Paired Owner Information," select “Get Serial Number." Do not put the accessory into a serial number read state. Select “Send Command." 15. Verify test completed with a red X. 16. Using iOS device B, launch the Find My app. Select “Items," swipe up, and select the “Identify Found Item." 17. Select the accessory and then select Continue. 18. Follow the instructions on the “Learn About This Item” page, follow the steps, and then select “Continue On Website." 19. Verify serial number listed matches the serial number printed on the accessory. |  |