| General Requirements Test Cases |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Case ID | Category | Test Case Description | Test Steps | Pass/Fail |
| TCG014 | (R2 addendum R1) 3. Find My device naming | When a Find My network-enabled accessory that advertises a non-Find My network Bluetooth payload is put in Bluetooth pairing mode by the end user, an updated device name that includes the suffix “- Find My” shall be presented in Bluetooth settings. This will indicate that location finding is enabled. The device name is limited to 14 characters total. When the owner device reconnects to the accessory, the accessory shall revert back to the original device name. | Perform this test case with the Find My app using the steps below. This test applies to accessories that advertise a non-Find My network Bluetooth payload. 1. Pair to the accessory over BT with iOS device A (using the BT menu or the accessory app). 2. Complete Find My Network Pairing with iOS device A. 3. Put the accessory into Bluetooth pairing mode. 4. Using a secondary iOS device B (on a different Apple ID account), go to the Bluetooth menu in Settings or go into the accessory app. Verify the discovered local name includes the suffix “- Find My." 5. Complete BT paring with iOS device B. 6. Verify on iOS device B that the name includes the suffix “- Find My." 7. From iOS device B, disconnect from the accessory from the Bluetooth settings or within the accessory app. 8. Using iOS device A, reconnect to the accessory. 9. Verify in the Bluetooth menu or the accessory app, that the local name reverted back to the original name without the suffix |  |
| TCG015 | (R2 addendum R1) 2. Accessories that advertise non-Find My network Bluetooth payload | Find My network-enabled accessories that advertise a Bluetooth payload separate to the Bluetooth payload advertised for the Find My network shall comply with the requirements below. - Bluetooth Classic accessories shall support GATT over Bluetooth Classic and Bluetooth LE. - Bluetooth LE accessories shall support • LE advertising extensions to advertise other services and payload using advertising sets. • Random resolvable addresses and periodic rotation of the addresses and Bluetooth payload used in advertising sets, rotating every 15 minutes. This will deter tracking of the accessory by a non-owner when it is in physical proximity to the owner. | Perform this test case with the Find My app and Packet Logger using the steps below. This test applies to accessories that advertise a non-Find My network Bluetooth Payload. 1. Pair to the accessory over Bluetooth. *make sure no other BT accessory is paired to the device. 2. Launch the Find My app. Under “Items," select Add Item, select Other Supported Item, and select Connect. Name the item and select Continue to complete the setup to add to Find My. 3. Connect Apple device to the Mac using a sync cable. 4. Launch Packet Logger, go to File, and select “New iOS Trace”. 5. Select the Device Filter pull down, select the BT address. 6. Communicate withe accessory using the Manufacture app. i.e. change configuration. image, etc. 7. Note down the BT address in the Packet Logger trace. 8. Wait 15 minutes. 9. Repeat steps 4-7 and verify accessory uses a completely different BT address from step 7. |  |