| Pairing/Discovery States Test Cases |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Case ID | Category | Test Case Description | Test Steps | Pass/Fail |
| TCP004 | 3.3.7 Reset | Verify that a factory reset deletes all Find My network (except AIS, FW version, Serial Number, SW authentication token, SW authentication UUID, Apple sever public keys, Q_A, Q_E). Verify the accessory has a physical mechanism to reset to default factory settings. Accessory must enable the user to set up the accessory using the Apple Find My app, both out of the box and after every factory reset, without requiring additional setup procedures. | Perform this test case with the Find My app and FMCA (Find My Certification Assistant) app using the steps below. 1. From iOS device A, launch the Find My app. Under “Items," select Add Item, select Other Supported Item, and select Connect. Name the item and select Continue to complete the setup to add to Find My. 2. Launch the FMCA app. Select the accessory under My Items. Please note: accessories that were unable to successfully pair to a device in Find My will be listed under the “My Items” section as “Find My Pairing issues." 3. Select “Accessory Information” under the Topics section. 4. Select “Get Accessory Information” under the Topics section to run the test. 5. Note down the following characteristic values - Product Data - Manufacture Name - Model Name - Accessory Category - Accessory Capabilities 6. Select “< Accessory Name” to go to the previous screen. 7. Select “Firmware” under the Topics section. 8. Select “Firmware Version” under the Topics section to run the test. 9. Note down the Firmware characteristic value. 10. Select “Find My Version” under the Topics section to run the test. 11. Verify Find My Version is set to 1.0.0. 12. Select “< Accessory Name” to go to the previous screen. 13. Select “Power Tests” under the Topics section. 14. Select “Battery Type” under the Topics section to run the test. 15. Note down the Battery Type characteristic response. 16. Select “Battery Level” under the Topics section to run the test. 17. Note down the Battery Level characteristic response. 18. Launch the Find My app. Select accessory name, scroll down, and select “Remove Item." 19. Verify the accessory has a physical way to reset to default factory settings. 20. Perform a factory reset. 21. Repeat steps 1-17 with a second iOS device B (on a different Apple ID account) and verify characteristic values remain the same. 22. While the accessory is paired to iOS device B, factory reset the accessory. 23. From iOS device A, launch the Find My app. Under “Items," select Add Item, select Other Supported Item, select Accessory, and select Connect. Name the item and select Continue. 24. When the pop up message “Could Not Continue” or “Cannot Add Item” appears, select Cancel. 25. From iOS device B, launch the Find My app, select accessory name, scroll down, and select “Remove Item.” 26. Place accessory into pairing mode. 27. From iOS device A, launch the Find My app. Under “Items," select Add Item, select Other Supported Item, and select Connect. Name the item and select Continue to complete the setup to add to Find My. |  |