| Pairing/Discovery States Test Cases |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Case ID | Category | Test Case Description | Test Steps | Pass/Fail |
| TCP005 | 4.6.3.4.7 Set max connections | Verify accessory sets the maxConnections to its maximum supported connections limit. | Perform this test case with the Find My app using the steps below. 1. From iOS device A, launch the Find My app. Under “Items," select Add Item, select Other Supported Item, and select Connect. Name the item and select Continue to complete the setup to add to Find My. 2. If the accessory supports more than 2 connections, continue to log into the same Apple ID account as device A with each additional iOS device. 3. From iOS device A, open the Find My app. Under “Items," select the accessory, and select Play Sound. *Applies to accessories that support sound maker. 4. Verify sound is heard from the accessory. *Applies to accessories that support sound maker. 5. Wait for sound to complete. 6. From a second iOS device B (on the same Apple ID account), launch the Find My app. Under “Items," select the accessory, and select Play Sound. *Applies to accessories that support sound maker. 7. Verify sound is heard from the accessory. *Applies to accessories that support sound maker. 8. Repeat steps 6-7 for each additional connection supported with an additional iOS device on the same Apple ID account. |  |
| TCP006 | 6.2.1 Pairing Mode / 3.3.6 Find My network pairing mode / 3.2 General | If an accessory supports both Find My and another Finder network, verify that the accessory only pairs to a single network at a time. Verify that the accessory requires explicit user intent (ex. press a button 3 times) to enable the Find My network pairing mode and that the accessory exits the pairing mode after 10 minutes. Please note: if the accessory is a Finder device, powering on or inserting a battery is sufficient as user intent. | Perform this test case with the Find My app, FMCA app, and manufacturer’s app using the steps below. 1. Power on the accessory. 2. Verify the accessory has a physical way to enable Find My network pairing mode. 3. Use manufacturer’s instructions to enable Find My network pairing mode. 4. Launch the Find My app. Under “Items," select Add Item, select Other Supported Item. Verify that the accessory is listed as an available item for pairing. 5. From the Android device, go to the Bluetooth menu and verify that the accessory is listed as an available item for pairing. 6. Wait 10+ minutes. 7. Launch the Find My app. Under “Items," select Add Item. Verify that the accessory is not listed as an available item for pairing. 8. Use the manufacturer’s instructions to enable pairing mode. 9. Launch the Find My app. Under “Items," select Add Item, select Other Supported Item. Verify that the accessory is listed as an available item for pairing. 10. From the Android device, go to the Bluetooth menu and verify that the accessory is listed as an available item for pairing. 11. Complete the pairing process with the Android device through the Bluetooth menu or Manufacturer’s app. 12. Launch the Find My app. Under “Items," select Add Item. Verify that the accessory is not listed as an available item for pairing. 13. From the iOS device, launch the FMCA app. Under “Discovered Items," verify the accessory is not listed as “Separated” or “Nearby." 14. From the Android device, disable Bluetooth. 15. From the iOS device, launch the FMCA app. Under “Discovered Items," verify the accessory is not listed as “Separated” or “Nearby." |  |