| Sound Maker Test Cases |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Case ID | Category | Test Case Description | Test Steps | Pass/Fail |
| TCSM001 | 4.6.3.4.1 Play Sound - owner control point | Verify Sound_start and Sound_stop opcode usage (Owner). | Perform this test case with the Find My app and FMCA (Find My Certification Assistant) app using the steps below. 1. Launch the Find My app. Under “Items," select Add Item, select Other Supported Item, and select Connect. Name the item and select Continue to complete the setup to add to Find My. 2. Launch the FMCA app. Select the accessory under My Items. Please note: accessories that were unable to successfully pair to a device in Find My will be listed under the “My Items” section as “Find My Pairing issues." 3. Select “Sound” under the Topics section. 4. Select “Stop Sound Validation” under the Topics section to run the test. 5. Verify results are passing. 6. Under the Results section, select “Save Results." |  |
| TCSM002 | 4.6.3.4.1 Play Sound - owner control point | Verify that the accessory responds with Sound_Completed message after the play sounds action has completed. | Perform this test case with the Find My app and FMCA (Find My Certification Assistant) app using the steps below. 1. Launch the Find My app. Under “Items," select Add Item, select Other Supported Item, and select Connect. Name the item and select Continue to complete the setup to add to Find My. 2. Launch the FMCA app. Select the accessory under My Items. Please note: accessories that were unable to successfully pair to a device in Find My will be listed under the “My Items” section as “Find My Pairing issues." 3. Select “Sound” under the Topics section. 4. Select “Play Sound Validation” under the Topics section to run the test. 5. Verify results are passing. 6. Under the Results section, select “Save Results." |  |
| TCSM003 | 4.6.3.4.1 Play Sound - owner control point | Verify Sound_start and Sound_stop opcode usage (Owner) after waking the iOS device from sleep. | Perform this test case with the Find My app and FMCA (Find My Certification Assistant) app using the steps below. 1. Launch the Find My app. Under “Items," select Add Item, select Other Supported Item, and select Connect. Name the item and select Continue to complete the setup to add to Find My. 2. Press the power button on the iOS device to place it to sleep. 3. Press the power button on the iOS device to wake up and then proceed to unlock. 4. Launch the FMCA app. Select the accessory under My Items. Please note: accessories that were unable to successfully pair to a device in Find My will be listed under the “My Items” section as “Find My Pairing issues." 5. In the FMCA app, select “Sound” under the Topics section. 6. Select “Stop Sound Validation” under the Topics section to run the test. Please note: you may have to select “Clear Results” before running the Stop Sound Validation test. 7. Verify results are passing. 8. Under the Results section, select “Save Results." |  |
| TCSM004 | 4.6.3.4.1 Play Sound - owner control point | Verify Sound_start and Sound_stop opcode usage (Owner) after power cycling the iOS device. | Perform this test case with the Find My app and FMCA (Find My Certification Assistant) app using the steps below. 1. Launch the Find My app. Under “Items," select Add Item, select Other Supported Item, and select Connect. Name the item and select Continue to complete the setup to add to Find My. 2. Power cycle the iOS. 3. Launch the FMCA app. Select the accessory under My Items. Please note: accessories that were unable to successfully pair to a device in Find My will be listed under the “My Items” section as “Find My Pairing issues." 4. Select “Sound” under the Topics section. 5. Select “Stop Sound Validation” under the Topics section to run the test. Please note: you may have to select “Clear Results” before running the Stop Sound Validation test. 6. Verify results are passing. 7. Under the Results section, select “Save Results." |  |
| TCSM005 | 4.6.3.4.1 Play Sound - owner control point | Verify Sound_start and Sound_stop opcode usage (Owner) after power cycling accessory. | Perform this test case with the Find My app and FMCA (Find My Certification Assistant) app using the steps below. A Debug control point enabled sample is required to perform this test case. 1. Launch the Find My app. Under “Items," select Add Item, select Other Supported Item, and select Connect. Name the item and select Continue to complete the setup to add to Find My. 2. Launch the FMCA app. Select the accessory under My Items. Please note: accessories that were unable to successfully pair to a device in Find My will be listed under the “My Items” section as “Find My Pairing issues." 3. Select “Sound” under the Topics section. 4. Select “Restart then Play+Stop” under the Topics section to run the test. 5. Verify results are passing. 6. Under the Results section, select “Save Results." |  |
| TCSM006 | 4.6.3.4.1 Play Sound - owner control point | Verify the accessory responds with Invalid_state ResponseStatus code (0x0001) when a Sound_Stop opcode is sent when no sound is in session. | Perform this test case with the Find My app and FMCA (Find My Certification Assistant) app using the steps below. 1. Launch the Find My app. Under “Items," select Add Item, select Other Supported Item, and select Connect. Name the item and select Continue to complete the setup to add to Find My. 2. Launch the FMCA app. Select the accessory under My Items. Please note: accessories that were unable to successfully pair to a device in Find My will be listed under the “My Items” section as “Find My Pairing issues." 3. Select “Sound” under the Topics section. 4. Select “Verify Stop with No Sound” under the Topics section to run the test. 5. Verify results are passing. 6. Under the Results section, select “Save Results." |  |