| Sound Maker Test Cases |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Case ID | Category | Test Case Description | Test Steps | Pass/Fail |
| TCSM007 | 4.6.3.4.1 Play Sound - owner control point | Verify Sound_start and Sound_stop opcode usage (Owner) after keyroll. | Perform this test case with the Find My app and FMCA (Find My Certification Assistant) app using the steps below. A Debug control point enabled sample is required to perform this test case. 1. From iOS device A, go to Settings, select Apple ID on top of page, Select Find My, and set “My Location” to this device. Please note: ensure that this Apple ID is only logged into iOS device A. 2. Launch the Find My app. Under “Items," select Add Item, select Other Supported Item, and select Connect. Name the item and select Continue to complete the setup to add to Find My. 3. Launch the FMCA app. Select the accessory under My Items. Please note: accessories that were unable to successfully pair to a device in Find My will be listed under the “My Items” section as “Find My Pairing issues." 4. Select “Key Management” under the Topics section. 5. Select “Set+Verify Key Rotations” under the Topics section to run the test. Make sure iOS device does not go to sleep while the test is running. Please note: you may have to select “Clear Results” before running the Set+Verify Key Rotations test. 6. Verify results are passing. 7. Launch the Find My app. 8. Under “Items," select the accessory, and select Play Sound. 9. Verify sound (or vibration) is heard. |  |
| TCSM008 | 4.6.3.4.1 Play Sound - owner control point | Verify the accessory responds to the Sound_Start opcode with Invalid_command ResponseStatus code if the accessory does not support the play sound procedure. | Perform this test case with the Find My app and FMCA (Find My Certification Assistant) app using the steps below. 1. Launch the Find My app. Under “Items," select Add Item, select Other Supported Item, and select Connect. Name the item and select Continue to complete the setup to add to Find My. 2. Launch the FMCA app. Select the accessory under My Items. Please note: accessories that were unable to successfully pair to a device in Find My will be listed under the “My Items” section as “Find My Pairing issues." 3. Select “Sound” under the Topics section. 4. Select “Stop Sound Validation” under the Topics section to run the test. 5. Under the Results section, select “Save Results." |  |
| TCSM009 | 7.2.2 Sound Maker | Verify the sound maker emits a sound with minimum 60 phon peak loudness as defined by ISO 532-1:2017. The loudness must be measured in free acoustic space substantially free of obstacles that would affect the pressure measurement. The loudness must be measured by a calibrated (to the Pascal) free field microphone 25 cm from the accessory suspended in free space. | Perform this test case with the Find My app using the steps below. 1. Launch the Find My app. Under “Items," select Add Item, select Other Supported Item, and select Connect. Name the item and select Continue to complete the setup to add to Find My. 2. Suspend the accessory in free space. 3. Place a microphone 25 cm from the accessory. 4. Launch the Find My app, select the accessory name, and select “Play Sound." 5. Verify the sound maker emits a sound with minimum 60 phon peak loudness as defined by ISO 532-1:2017. |  |