| Unwanted Tracking Test Cases |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Case ID | Category | Test Case Description | Test Steps | Pass/Fail |
| TCUT002 | 2.7.4 Separated / 7.3 Implementation | For accessories that support motion-triggered UT sound alerts, verify that motion detection is enabled while in a Separated state. | Perform this test case with the Find My app and FMCA (Find My Certification Assistant) app using the steps below. A Debug control point enabled sample is required to perform this test case. Please note: execute test case TCP002 and verify it passes prior to starting this test case. 1. From iOS device, launch the Find My app. Under “Items," select Add Item, select Other Supported Item, and select Connect. Name the item and select Continue to complete the setup to add to Find My. 2. Launch the FMCA app. Select the accessory under My Items. Please note: accessories that were unable to successfully pair to a device in Find My will be listed under the “My Items” section as “Find My Pairing issues." 3. Select “Debug” under the Topics section. 4. Select “Reduce Separated Motion Timers” under the Topics section to run the test. 5. Select “Turn Off Bluetooth." 6. Wait until the accessory transitions into separated mode. 7. From a second iOS device (on a different Apple ID account), launch the FMCA app. Under “Discovered Items," verify accessory is listed as “Separated.” Wait 2 minutes before proceeding to the next step. 8. On the second device start a stop watch timer. Perform steps 9-10 within a 20 second window after first motion is detected. 9. Move the accessory ± 10° orientation along any two axis and verify sound (or vibration) is heard when motion is detected. Wait until the sound alert had completed before proceeding to the next step. 10. Continue to repeat step 9 until the 20 second window is complete. 11. Verify multiple play sound alerts are heard during the 20 second window. 12. Wait for the timer to hit the 1 minute mark and move the accessory ± 10° orientation along any two axis. 13. Verify no sound (or vibration) is heard when motion is detected. 14. Wait for the timer to hit the 3 minute mark and move the accessory ± 10° orientation along any two axis. 15. Verify no sound (or vibration) is heard when motion is detected. 16. Wait for the timer to hit the 4 minute mark and move the accessory ± 10° orientation along any two axis. 17. Verify no sound (or vibration) is heard when motion is detected. 18. Wait until the 5 minute mark has elapsed. 19. Move the accessory ± 10° orientation along any two axis continuously for 30 seconds. 20. Verify sound (or vibration) is heard when motion is detected. Wait until sound maker had completed before proceeding to the next step. 21. Move the accessory ± 10° orientation along any two axis again. 22. Verify no sound (or vibration) is heard when motion is detected. 23. Enable Bluetooth from the iOS settings. 24. Select “Reset Separated Motion Timers." |  |
| TCUT003 | 7.3 Implementation | Verify the steps for Learn About This Item and that the illustration matches the location on the accessory in the Find My app. Verify the steps for Instructions to Disable Item and that the illustration matches the location on the accessory to disable in the Find My app. | Perform this test case with the Find My app and using the steps below. 1. From iOS device A, launch the Find My app. Under “Items," select Add Item, select Other Supported Item, and select Connect. Name the item and select Continue to complete the setup to add to Find My. 2. Enable Lost Mode and complete steps to Activate. 3. Disable Bluetooth on device A from the iOS settings. 4. From a second iOS device B (on a different Apple ID account), launch the Find My app. Select Items, swipe up, and select “Identify Found Item”. 5. Select the accessory and select Continue. 6. Follow instructions under “Learn About This Item." 7. Verify that the illustration and steps point to the correct location on the accessory. 8. Select “Continue to Website”. 9. Verify that the serial number listed matches the serial number on the accessory. 10. Select the “Instructions to Disable” link. 11. Follow the “How to disable” instructions. 12. Verify that the illustration and steps point to the correct location on the accessory. 13. Verify that the serial number listed matches the serial number on the accessory. 14. From iOS device A, Enable Bluetooth, launch the Find My app, and Turn Off Lost Mode. |  |