| Firmware Updates Test Cases |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Case ID | Category | Test Case Description | Test Steps iOS versions older than iOS 17 | Pass/Fail |
| TCFW004 | 10. Firmware Update | Verify the Find My accessory can handle an interruption during the firmware update process when the accessory power cycled (if applicable). | Perform this test case with the Find My app and FMCA (Find My Certification Assistant) app using the steps below. Applies to accessories that support Firmware Update by UARP. Please download and install the Find My Network Configuration Profile from the MFi Portal Content Center. 1. Launch the Find My app. Under “Items," select Add Item, select Other Supported Item, and select Connect. Name the item and select Continue to complete the setup to add to Find My. 2. Launch the FMCA app. Select the accessory under My Items. Please note: accessories that were unable to successfully pair to a device in Find My will be listed under the “My Items” section as “Find My Pairing issues." 3. Select “Firmware” under the Topics section. 4. Select “Firmware Version” under the Topics section to run the test. 5. Note down firmware revision and select Clear Results. 6. Select the accessory name on top left to return to the previous screen. 7. Select “Firmware Settings." 8. Select “Choose a File…." 9. Select a .UARP file that is stored locally on the iOS device. i.e Files folder 10. Select “Update available." 11. Select “Update” to the pop up prompt. 12. Power cycle the accessory. i.e. remove battery or power off 13. Verify the accessory reconnects with the iOS device. 14. From the Find My app, select Play Sound. 15. Verify sound is heard from the accessory. 16. From the FMCA app, select “Firmware” under the Topics section. 17. Select “Firmware Version” under the Topics section to run the test. 18. Verify the Firmware version remained the same from step 5. |  |
| TCFW005 | 10. Firmware Update | Verify the Find My accessory can handle an interruption during the firmware update process when the Apple device power cycled. | Perform this test case with the Find My app and FMCA (Find My Certification Assistant) app using the steps below. Applies to accessories that support Firmware Update by UARP. Please download and install the Find My Network Configuration Profile from the MFi Portal Content Center. 1. Launch the Find My app. Under “Items," select Add Item, select Other Supported Item, and select Connect. Name the item and select Continue to complete the setup to add to Find My. 2. Launch the FMCA app. Select the accessory under My Items. Please note: accessories that were unable to successfully pair to a device in Find My will be listed under the “My Items” section as “Find My Pairing issues." 3. Select “Firmware” under the Topics section. 4. Select “Firmware Version” under the Topics section to run the test. 5. Note down firmware revision and select Clear Results. 6. Select the accessory name on top left to return to the previous screen. 7. Select “Firmware Settings." 8. Select “Choose a File…." 9. Select a .UARP file that is stored locally on the iOS device. i.e Files folder 10. Select “Update available." 11. Select “Update” to the pop up prompt. 12. Power cycle the iOS device. 13. Launch the Find My app and verify the accessory reconnects with the iOS device. 14. Select Play Sound. 15. Verify sound is heard from the accessory. 16. From the FMCA app, select “Firmware” under the Topics section. 17. Select “Firmware Version” under the Topics section to run the test. 18. Verify the Firmware version remained the same from step 5. |  |
| TCFW006 | 10. Firmware Update | Verify the Find My accessory can handle an interruption during the firmware update process when Bluetooth is disconnected. | Perform this test case with the Find My app and FMCA (Find My Certification Assistant) app using the steps below. Applies to accessories that support Firmware Update by UARP. Please download and install the Find My Network Configuration Profile from the MFi Portal Content Center. 1. Launch the Find My app. Under “Items," select Add Item, select Other Supported Item, and select Connect. Name the item and select Continue to complete the setup to add to Find My. 2. Launch the FMCA app. Select the accessory under My Items. Please note: accessories that were unable to successfully pair to a device in Find My will be listed under the “My Items” section as “Find My Pairing issues." 3. Select “Firmware” under the Topics section. 4. Select “Firmware Version” under the Topics section to run the test. 5. Note down firmware revision and select Clear Results. 6. Select the accessory name on top left to return to the previous screen. 7. Select “Firmware Settings." 8. Select “Choose a File…." 9. Select a .UARP file that is stored locally on the iOS device. i.e Files folder 10. Select “Update available." 11. Select “Update” to the pop up prompt. 12. Disable Bluetooth from the iOS settings. 13. Enable Bluetooth from the iOS settings. 14. Verify the accessory reconnects with the iOS device. 15. From the Find My app, select Play Sound. 16. Verify sound is heard from the accessory. 17. From the FMCA app, select “Firmware” under the Topics section. 18. Select “Firmware Version” under the Topics section to run the test. 19. Verify the Firmware version remained the same from step 5. |  |