| Beacon States Test Cases |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Case ID | Category | Test Case Description | Test Steps | Pass/Fail |
| TCB009 | 4.6.3.4.3 Set nearby timeout | Verify the transition from nearby state to separated state when the set NearbyTimeOut parameter has reached 0 seconds. | Perform this test case with the Find My app and FMCA (Find My Certification Assistant) app using the steps below. Please note: execute test case TCP002 and verify it passes prior to starting this test case. A Debug control point enabled sample is required to perform this test case. 1. From iOS device A, launch the Find My app. Under “Items," select Add Item, select Other Supported Item, and select Connect. Name the item and select Continue to complete the setup to add to Find My. 2. Launch the FMCA app. Select the accessory under My Items. Please note: accessories that were unable to successfully pair to a device in Find My will be listed under the “My Items” section as “Find My Pairing issues." 3. Select “Connectivity” under the Topics section. 4. Select “Set Nearby Timeout” under the Topics section to run the test. 5. Select “Tap to Turn Off Bluetooth." 6. Verify “Tap to Turn Off Bluetooth” completed with a green check. 7. From a second iOS device B (on a different Apple ID account), launch the FMCA app. 8. Under “Discovered Items," verify accessory is listed as “Nearby," and then select the accessory. Scroll down and verify the Service Availability section is populated before proceeding to the next step. 9. Select the “Share” icon on the top right corner. 10. Select iOS device A’s name to connect and then select “Done." 11. From iOS device A, verify “Confirmed Near Owner” completed with a green check. 12. From iOS device B, under “Discovered Items," verify accessory is listed as “Separated," and then select the accessory. 13. Verify the following information is displayed under Accessory Information: - Product Data - Manufacturer Name - Model Name - Accessory Category - Accessory Capabilities - Firmware Version - Find My Version - Battery Type - Battery Level 14. Under “Non Owner Control Point," select “Present Unwanted Tracking Alert." 15. Select “Sound Start” and confirm sound is played from the accessory. 16. When the “Sound Stop” font changes gray to purple, select “Sound Stop," and confirm the sound stopped playing. *Applies to accessories that support sound maker. 17. Verify both tests completed with a green check. 18. Under “Paired Owner Information," select Get Primary Key. 19. Verify test completed with a green check. 20. Select “Get iCloud ID." 21. Verify test completed with a green check. 22. For accessories that support serial number lookup over BLE. Perform the following step. Select “Get Serial Number." For accessories that support serial number lookup over NFC only, skip to step 24. 23. Verify test completed with a green check. 24. Under the "Configuration Control Point”, select “Start Sound." *Applies to accessories that support sound maker. 25. Verify test completed with a green check and that no sound (or vibration) is heard from the accessory. 26. Select “Unpair." 27. Verify test completed with a green check. 28. Select the “Share” icon on the top right corner. 29. Select iOS device A’s name to connect and then select “Done." 30. From iOS device A, verify “Confirmed Separated” completed with a green check. 31. Select “Close” on the top left corner. 32. Under the Results section, select “Save Results." |  |