| Beacon States Test Cases |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Case ID | Category | Test Case Description | Test Steps | Pass/Fail |
| TCB010 | 4.9 Service availability | While the accessory in a Connected (Owner) state, verify the following Services and Characteristics are available. Find My Network Service - Configuration control point - Paired owner information control point Firmware Update (If supported) - All characteristics Accessory Information Service - All characteristics | Perform this test case with the Find My app and FMCA (Find My Certification Assistant) app using the steps below. 1. Launch the Find My app. Under “Items," select Add Item, select Other Supported Item, and select Connect. Name the item and select Continue to complete the setup to add to Find My. 2. Launch the FMCA app. Select the accessory under My Items. Please note: accessories that were unable to successfully pair to a device in Find My will be listed under the “My Items” section as “Find My Pairing issues." 3. Select “Sound” under the Topics section. 4. Select “Non-Owner Sound Availability” under the Topics section to run the test. 5. Verify results are passing. 6. Select “< Accessory Name” to go to the previous screen. 7. Select “< Products” to go to the previous screen. 8. Swipe down from the top of the screen to refresh the FMCA app. 9. Under “Discovered Items," verify accessory is listed as “Nearby," and then select the accessory. 10. Verify Accessory Information is populated. 11. Under Service Availability, verify the following characteristics are set as following Pairing - Unavailable* Configuration - Available Non Owner - Unavailable* Paired Owner Info - Available Debug - Available in debug version only Firmware Update - Available (If supported) * If accessory is not able to dynamically update the Pairing and Non Owner characteristics under the Service Availability, please verify results are passing after performing step 4 and test case TCB004 passed successfully. 12. Select “Save Results” on the top right corners. |  |