| Pairing/Discovery States Test Cases |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Case ID | Category | Test Case Description | Test Steps | Pass/Fail |
| TCP001 | 6.2.1 Pairing Mode / 3.3.6 Find My network pairing mode | Verify that the accessory requires explicit user intent (ex. press a button 3 times) to enable the Find My network pairing mode and that the accessory exits the pairing mode after 10 minutes. Please note: if the accessory is a Finder device, powering on or inserting a battery is sufficient as user intent. | Perform this test case with the Find My app using the steps below. 1. Power on the accessory. 2. Verify the accessory has a physical way to enable Find My network pairing mode. 3. Use manufacturer’s instructions to enable Find My network pairing mode. 4. Launch the Find My app. Under “Items," select Add Item, select Other Supported Item. Verify accessory is listed as an available item for pairing. 5. Wait 10+ minutes. 6. Launch the Find My app. Under “Items," select Add Item. Verify accessory is not listed as an available item for pairing. 7. Use manufacturer’s instructions to enable Find My network pairing mode. 8. Launch the Find My app. Under “Items," select Add Item, select Other Supported Item. Verify accessory is listed as an available item for pairing. |  |
| TCP002 | 2.7.2 Connected | Verify that the accessory enters Connected state once Find My network pairing is successfully completed with the Owner device. | Perform this test case with the Find My app and FMCA (Find My Certification Assistant) app using the steps below. A Debug control point enabled sample is required to perform this test case. 1. Launch the Find My app. Under “Items," select Add Item, select Other Supported Item, and select Connect. Name the item and select Continue to complete the setup to add to Find My. 2. Launch the FMCA app. Select the accessory under My Items. 3. Select “Connectivity” under the Topics section. 4. Select “Verify Connection” under the Topics section to run the test. 5. Verify results are passing. 6. Select “Connection Status” under the Topics section to run the test. 7. Verify results are passing. 8. Under the Results section, select “Save Results." |  |
| TCP003 | 4.6.3.8.2 Get iCloud Identifier | If the Find My network pairing is complete, the accessory shall respond with Get_iCloud_Identifier_Response with the iCloud identifier. | Perform this test case with the Find My app and FMCA (Find My Certification Assistant) app using the steps below. 1. Launch the Find My app. Under “Items," select Add Item, select Other Supported Item, and select Connect. Name the item and select Continue to complete the setup to add to Find My. 2. Launch the FMCA app. Select the accessory under My Items. Please note: accessories that were unable to successfully pair to a device in Find My will be listed under the “My Items” section as “Find My Pairing issues." 3. Select “Accessory Information” under the Topics section. 4. Select “Get iCloud Identifier” under the Topics section to run the test. 5. Verify results are passing and that the displayed Apple ID matches. 6. Under the Results section, select “Save Results." |  |